Behind the Scenes

The Complete Arduino Compilation Process

From Code to Running Microcontroller

# Table of Contents

# 1. The Complete Journey: Overview

When you click the Upload button in Arduino IDE, a complex series of processes happens in milliseconds. This document explains every step from your .ino file to a running program on the microcontroller.

## 1.1 The Big Picture

- Arduino IDE preprocesses your .ino file into C++ code

- The AVR-GCC compiler translates C++ to assembly language

- The assembler converts assembly to machine code (object files)

- The linker combines object files into a single executable

- objcopy creates a HEX file for uploading

- avrdude uploads the HEX file via USB to the bootloader

- The bootloader writes the program to flash memory

- The microcontroller resets and runs your program

Each of these steps involves sophisticated tools and processes. Let's explore them in detail.

# 2. Stage 1: Arduino Preprocessing

## 2.1 Your Original .ino File

Let's say you write this simple Arduino sketch:

void setup() {

Serial.begin(9600);

pinMode(13, OUTPUT);

}

void loop() {

digitalWrite(13, HIGH);

delay(1000);

digitalWrite(13, LOW);

delay(1000);

}

## 2.2 What Arduino IDE Does

The Arduino IDE performs several transformations:

### Step 1: Add Function Prototypes

Arduino automatically declares your functions at the top:

void setup();

void loop();

### Step 2: Add #include <Arduino.h>

This is automatically added at the very beginning:

#include <Arduino.h>

### Step 3: Create main() Function

Arduino wraps your setup() and loop() in a main() function:

int main(void) {

init();         // Initialize timers, ADC, etc.

setup();        // Your setup()

for (;;) {      // Infinite loop

loop();       // Your loop()

if (serialEventRun) serialEventRun();

}

return 0;

}

### Step 4: The Complete Preprocessed File

After preprocessing, your sketch becomes a complete C++ file:

#include <Arduino.h>

void setup();

void loop();

void setup() {

Serial.begin(9600);

pinMode(13, OUTPUT);

}

void loop() {

digitalWrite(13, HIGH);

delay(1000);

digitalWrite(13, LOW);

delay(1000);

}

# 3. Stage 2: The Arduino.h Include Chain

## 3.1 What's Inside Arduino.h

Arduino.h is located at:

hardware/arduino/avr/cores/arduino/Arduino.h

It includes essential AVR and Arduino headers:

#include <stdlib.h>

#include <stdbool.h>

#include <string.h>

#include <math.h>

#include <avr/pgmspace.h>   // Flash memory functions

#include <avr/io.h>         // Hardware register definitions

#include <avr/interrupt.h>  // Interrupt macros

// Arduino-specific includes

#include "binary.h"

#include "WCharacter.h"

#include "WString.h"

#include "HardwareSerial.h"

#include "pins_arduino.h"

## 3.2 The avr/io.h Include Chain

The avr/io.h file determines which microcontroller you're using:

// Inside avr/io.h

#if defined (__AVR_ATmega328P__)

# include <avr/iom328p.h>

#elif defined (__AVR_ATmega2560__)

# include <avr/iom2560.h>

#elif defined (__AVR_ATmega32U4__)

# include <avr/iom32u4.h>

#else

# error "Device not supported"

#endif

## 3.3 What's in iom328p.h

This file contains all hardware register definitions:

// Register addresses (memory-mapped I/O)

#define PORTB  _SFR_IO8(0x05)   // Port B Output Register

#define DDRB   _SFR_IO8(0x04)   // Port B Direction Register

#define PINB   _SFR_IO8(0x03)   // Port B Input Register

#define SPCR   _SFR_IO8(0x2C)   // SPI Control Register

#define SPSR   _SFR_IO8(0x2D)   // SPI Status Register

#define SPDR   _SFR_IO8(0x2E)   // SPI Data Register

// Bit positions

#define PB5    5                 // Pin 5 in Port B

#define SPE    6                 // SPI Enable bit

#define SPIE   7                 // SPI Interrupt Enable

// Interrupt vectors

#define SPI_STC_vect_num   17

#define SPI_STC_vect       _VECTOR(17)

# 4. Stage 3: C Preprocessor (cpp)

## 4.1 What the Preprocessor Does

The C preprocessor (cpp) runs before compilation and:

- Processes all #include directives

- Expands all #define macros

- Handles #if, #ifdef, #ifndef conditional compilation

- Removes comments

- Produces a single massive .cpp file with everything expanded

## 4.2 Macro Expansion Example

Your code:

pinMode(13, OUTPUT);

After preprocessor expansion:

// pinMode is a function in wiring_digital.c

// OUTPUT is defined as 0x1 in Arduino.h

pinMode(13, 0x1);

## 4.3 Register Access Macro Expansion

When you write:

SPCR = (1 << SPE);

The preprocessor expands it to:

// _SFR_IO8(0x2C) expands to:

(*(volatile uint8_t *)((0x2C) + 0x20)) = (1 << 6);

This is direct memory access to address 0x4C (0x2C + 0x20 offset).

# 5. Stage 4: Compilation to Assembly

## 5.1 The AVR-GCC Compiler

The compiler command looks like this:

avr-g++ -c -g -Os -w -std=gnu++11 -fpermissive \

-fno-exceptions -ffunction-sections -fdata-sections \

-mmcu=atmega328p -DF_CPU=16000000L \

-DARDUINO=10819 -DARDUINO_AVR_UNO \

-I"cores/arduino" -I"variants/standard" \

sketch.cpp -o sketch.cpp.o

Key flags explained:

- -mmcu=atmega328p → Target the ATmega328P microcontroller

- -DF_CPU=16000000L → CPU frequency is 16MHz

- -Os → Optimize for size (important for limited flash memory)

- -ffunction-sections → Put each function in its own section (for linker optimization)

## 5.2 C++ to Assembly Translation

Your C++ code:

digitalWrite(13, HIGH);

Compiles to assembly (simplified):

; Set bit 5 of PORTB (pin 13 on Uno)

sbi  0x05, 5    ; Set Bit in I/O register

Or more explicitly:

in   r24, 0x05   ; Read PORTB into register r24

ori  r24, 0x20   ; OR with 0b00100000 (bit 5)

out  0x05, r24   ; Write back to PORTB

## 5.3 SPI Example Compilation

C code:

SPCR = (1 << SPE) | (1 << SPIE);

Assembly:

ldi  r24, 0xC0   ; Load 0b11000000 into r24

out  0x2C, r24   ; Write to SPCR (address 0x2C)

# 6. Stage 5: Assembly to Machine Code

## 6.1 The Assembler (avr-as)

The assembler converts assembly instructions to binary machine code:

| Assembly | Machine Code | Description |
| --- | --- | --- |
| sbi 0x05, 5 | 9A 2D | Set bit 5 in I/O 0x05 |
| ldi r24, 0xC0 | E8 CC | Load immediate 0xC0 |
| out 0x2C, r24 | B8 8C | Output to I/O 0x2C |

## 6.2 Object Files (.o)

The assembler creates object files (.o) containing:

- Machine code for each function

- Symbol table (function names, addresses)

- Relocation information (addresses to fix during linking)

- Debug information (if -g flag used)

Object files created:

- sketch.cpp.o (your sketch)

- wiring_digital.o (digitalWrite, pinMode, etc.)

- HardwareSerial.o (Serial communication)

- main.o (main function)

- + many more Arduino core files

# 7. Stage 6: Linking

## 7.1 The Linker (avr-gcc)

The linker combines all .o files into a single executable:

avr-gcc -Os -Wl,--gc-sections -mmcu=atmega328p \

-o sketch.elf \

sketch.cpp.o wiring_digital.o HardwareSerial.o ... \

-lm  # Link math library

## 7.2 What the Linker Does

- Combines all object files

- Resolves function addresses (when you call digitalWrite(), it finds the actual address)

- Removes unused code (--gc-sections flag)

- Arranges code in flash memory starting at address 0x0000

- Places data in RAM and EEPROM

- Creates the final .elf file

## 7.3 Memory Layout

The linker organizes memory according to the ATmega328P layout:

| Memory Type | Size | Address Range | Contents |
| --- | --- | --- | --- |
| Flash | 32KB | 0x0000-0x7FFF | Program code |
| SRAM | 2KB | 0x0100-0x08FF | Variables, stack |
| EEPROM | 1KB | 0x0000-0x03FF | Persistent data |
| Bootloader | 0.5KB | 0x7E00-0x7FFF | Upload code |

Your program starts at 0x0000, and the bootloader lives at the end of flash (0x7E00).

# 8. Stage 7: Creating the HEX File

## 8.1 avr-objcopy

The .elf file is converted to Intel HEX format:

avr-objcopy -O ihex -R .eeprom sketch.elf sketch.hex

## 8.2 Intel HEX Format

The HEX file is a text file containing hexadecimal data:

:100000000C945D000C9485000C9485000C94850084

:100010000C9485000C9485000C9485000C9485004C

:100020000C9485000C9485000C9485000C9485003C

:10003000C0E0D0E000E00BBF0895AA1ABB1ABA2C18

Each line format:

- :10 - Start code and byte count (16 bytes)

- 0000 - Address where data goes

- 00 - Record type (00 = data)

- [data bytes] - The actual program code

- 84 - Checksum

# 9. Stage 8: Uploading with avrdude

## 9.1 The avrdude Command

Arduino IDE uses avrdude to upload:

avrdude -C avrdude.conf \

-v -p atmega328p \

-c arduino \

-P COM3 \

-b 115200 \

-D -U flash:w:sketch.hex:i

Flag meanings:

- -p atmega328p → Target chip

- -c arduino → Use Arduino bootloader protocol

- -P COM3 → Serial port (COM3 on Windows, /dev/ttyUSB0 on Linux)

- -b 115200 → Baud rate for communication

- -D → Don't erase chip before writing

- -U flash:w:sketch.hex:i → Write HEX file to flash memory

## 9.2 The Upload Process

- Arduino IDE triggers DTR signal on USB-Serial chip

- DTR pulse resets the Arduino (pulls RESET pin low)

- Arduino boots and bootloader starts running

- Bootloader waits ~2 seconds for upload

- avrdude sends HEX file data via serial

- Bootloader writes each block to flash memory

- Bootloader verifies the written data

- Upload complete, bootloader jumps to address 0x0000

- Your program starts running!

# 10. The Bootloader: A Program Within a Program

## 10.1 What is a Bootloader?

The bootloader is a small program (512 bytes) pre-installed in flash memory that:

- Runs when the Arduino resets

- Listens on serial port for upload commands

- Writes new programs to flash memory

- Then launches your program

## 10.2 Bootloader Memory Location

On ATmega328P:

- Bootloader starts at: 0x7E00 (last 512 bytes)

- Your program uses: 0x0000 to 0x7DFF (31.5 KB)

- Protected by BOOTSZ fuses

## 10.3 How Arduino Boots

- Power on or reset occurs

- CPU starts executing from bootloader (fuse configuration)

- Bootloader checks if DTR pulse occurred recently

- If yes: Enter programming mode, wait for data

- If no: Jump to 0x0000 and run user program

## 10.4 STK500 Protocol

The bootloader uses the STK500 protocol for communication:

avrdude → 0x30 0x20  (Get sync)

bootloader → 0x14 0x10  (In sync)

avrdude → 0x55  (Load address)

bootloader → 0x14 0x10  (OK)

avrdude → [data]  (Write flash)

bootloader → 0x14 0x10  (OK)

# 11. Runtime: How Your Program Executes

## 11.1 CPU Startup Sequence

- CPU resets, program counter = 0x0000

- Reset vector points to init() function

- init() configures timers, ADC, PWM

- main() function is called

- main() calls your setup()

- main() enters infinite loop calling loop()

## 11.2 The Hidden main() Function

Remember, Arduino creates this for you:

int main(void) {

init();  // Hardware initialization

initVariant();  // Board-specific init

#if defined(USBCON)

USBDevice.attach();

#endif

setup();  // YOUR setup()

for (;;) {  // Infinite loop

loop();  // YOUR loop()

if (serialEventRun) serialEventRun();

}

return 0;  // Never reached

}

# 12. Hardware Interrupts: Deep Dive

## 12.1 What Happens When SPI_STC_vect Fires

Let's trace what happens when SPI completes a byte transfer:

- SPI hardware finishes shifting 8 bits

- SPI hardware sets SPIF flag in SPSR register

- If SPIE bit is set in SPCR, interrupt is triggered

- CPU saves current instruction address to stack

- CPU saves SREG (status register) to stack

- CPU jumps to interrupt vector table

- Vector table entry #17 points to your ISR

- Your ISR(SPI_STC_vect) code executes

- ISR ends with 'reti' instruction

- CPU restores SREG from stack

- CPU restores return address from stack

- Execution continues where it was interrupted

## 12.2 The Interrupt Vector Table

At the very start of flash memory (0x0000), there's a jump table:

; Address  Vector   Description

0x0000:  jmp  RESET       ; Reset

0x0002:  jmp  INT0_ISR    ; External Interrupt 0

0x0004:  jmp  INT1_ISR    ; External Interrupt 1

...

0x0022:  jmp  SPI_STC_ISR ; SPI Transfer Complete (vector 17)

...

## 12.3 How ISR Macro Works

When you write:

ISR(SPI_STC_vect) {

byte data = SPDR;

}

The preprocessor expands it to:

void __vector_17(void) __attribute__((signal, used, externally_visible));

void __vector_17(void) {

byte data = SPDR;

}

The compiler generates:

__vector_17:

push r1        ; Save registers

push r24

in   r1, 0x3F  ; Save SREG

push r1

in   r24, 0x2E ; Read SPDR

; ... your code ...

pop  r1        ; Restore registers

out  0x3F, r1  ; Restore SREG

pop  r24

pop  r1

reti           ; Return from interrupt

# 13. Memory-Mapped I/O: How Registers Work

## 13.1 The ATmega328P Memory Map

The AVR uses memory-mapped I/O. This means hardware registers appear as memory locations:

| Address Range | Purpose | Examples |
| --- | --- | --- |
| 0x0000-0x001F | CPU Registers | r0-r31 |
| 0x0020-0x005F | I/O Registers | PORTB, SPCR, etc. |
| 0x0060-0x00FF | Extended I/O | Timers, USART |
| 0x0100-0x08FF | SRAM | Your variables |

## 13.2 How SPCR = (1 << SPE) Works

Step by step:

- SPCR is defined as: #define SPCR _SFR_IO8(0x2C)

- _SFR_IO8(0x2C) expands to: (*(volatile uint8_t *)(0x2C + 0x20))

- This creates a pointer to address 0x4C

- SPE is defined as: #define SPE 6

- (1 << SPE) becomes: (1 << 6) = 0b01000000 = 0x40

- The assignment writes 0x40 to memory address 0x4C

- The SPI hardware is physically connected to this memory location

- The SPI hardware reads the value and enables itself

## 13.3 Volatile Keyword

Notice the 'volatile' keyword. This tells the compiler:

- Don't optimize away reads/writes to this address

- The value can change outside program control (by hardware)

- Always read from memory, never cache in CPU register

# 14. Complete Timeline: Click to Running

Let's trace the complete journey from clicking Upload to your LED blinking:

### Time T+0ms: You Click Upload

- Arduino IDE saves your .ino file

- IDE creates a temporary build directory

### Time T+50ms: Preprocessing

- IDE adds #include <Arduino.h>

- IDE generates function prototypes

- IDE wraps code in main()

- C preprocessor expands all #includes

- Preprocessor expands all #defines

### Time T+200ms: Compilation

- avr-g++ compiles sketch to assembly

- avr-g++ compiles Arduino core files

- avr-as assembles to machine code

- Multiple .o object files created

### Time T+400ms: Linking

- avr-gcc links all .o files

- Resolves function addresses

- Removes unused code

- Creates sketch.elf file

### Time T+450ms: Creating HEX

- avr-objcopy converts .elf to .hex

- Intel HEX format created

- Size calculations displayed

### Time T+500ms: Upload Starts

- avrdude starts

- DTR signal toggles, Arduino resets

- Bootloader starts (2 second timeout)

- avrdude syncs with bootloader

### Time T+1000ms: Programming Flash

- avrdude sends HEX data in chunks

- Bootloader writes each chunk to flash

- Bootloader verifies each chunk

- Progress shown in IDE

### Time T+3000ms: Upload Complete

- Final verification

- avrdude disconnects

- Bootloader jumps to 0x0000

### Time T+3010ms: Your Program Starts

- CPU starts executing at 0x0000

- Reset vector jumps to init()

- init() configures hardware

- main() is called

- Your setup() runs

- Infinite loop starts calling loop()

- LED starts blinking!

# 15. The Complete Toolchain

## 15.1 Tools Involved

| Tool | Purpose | Input → Output |
| --- | --- | --- |
| Arduino IDE | Preprocessing | .ino → .cpp |
| cpp | C Preprocessor | .cpp → expanded .cpp |
| avr-g++ | Compiler | .cpp → .s (assembly) |
| avr-as | Assembler | .s → .o (object) |
| avr-gcc | Linker | Multiple .o → .elf |
| avr-objcopy | Format Converter | .elf → .hex |
| avrdude | Programmer | .hex → Flash memory |

## 15.2 Where These Tools Live

On Windows:

C:\Program Files (x86)\Arduino\hardware\tools\avr\bin\

On Linux:

/usr/share/arduino/hardware/tools/avr/bin/

On Mac:

/Applications/Arduino.app/Contents/Java/hardware/tools/avr/bin/

# 16. Summary: The Complete Picture

Now you understand the complete journey:

- Your simple .ino file is preprocessed into full C++

- Arduino.h automatically includes all necessary headers

- avr/io.h intelligently includes the correct chip header (iom328p.h)

- The C preprocessor expands all macros and includes

- The compiler translates C++ to assembly language

- The assembler converts assembly to machine code

- The linker combines everything into a single executable

- avr-objcopy creates the HEX file for uploading

- avrdude uploads via the bootloader

- The bootloader writes to flash memory

- Your program runs, with interrupts handled by hardware

## The Magic Revealed

What seems like simple Arduino code:

digitalWrite(13, HIGH);

Actually becomes:

- Preprocessed into C++

- Expanded through multiple header files

- Compiled to AVR assembly

- Assembled to machine code: 9A 2D

- Linked with Arduino core

- Converted to HEX format

- Uploaded to flash memory

- Executed as direct hardware register manipulation

All of this happens in just a few seconds, hidden behind the friendly Arduino interface!

Understanding this process gives you the power to:

- Debug compilation errors more effectively

- Optimize your code for size and speed

- Write more efficient interrupt handlers

- Understand memory limitations

- Work with direct register manipulation when needed

- Appreciate the sophisticated toolchain that makes Arduino programming easy

You now know what happens behind the scenes!